HRTF Humanoid Robot Test Framework Software Requirements Specification

| Version | 1.0 — Initial draft |
| --- | --- |
| Date | April 2026 |
| Status | Draft — for internal review and stakeholder feedback |
| Platform target | ROS 2 / Python-first, Simulink-compatible back-end |
| Primary audience | Systems / test engineers at humanoid robot companies; MathWorks product stakeholders |

1. Introduction

## 1.1  Purpose

This document specifies the functional and non-functional requirements for the Humanoid Robot Test Framework (HRTF) — an open-source test and validation platform designed specifically for humanoid robot systems. HRTF targets the gap between generic software testing tools (e.g., pytest) and general-purpose robotics simulators (e.g., NVIDIA Isaac Sim, MuJoCo): it provides structured, reusable, physics-aware test cases for the full humanoid control stack, from actuator-level torque compliance through whole-body locomotion stability and AI policy regression.

## 1.2  Problem statement

Humanoid robot companies currently have no standardised test and validation toolchain. Each organisation independently builds bespoke Python scripts and shell-based CI hooks that:

- Cannot assert continuous-time physical behaviour (e.g., 'CoM height must remain above 0.75 m for the 10 s following a lateral disturbance')

- Provide no baseline management for AI/RL policy regression after retraining

- Produce no machine-readable compliance evidence against emerging standards (ISO 25785-1)

- Require complete rewriting when a team moves from simulation to hardware-in-the-loop

HRTF solves these gaps with a ROS 2 / Python-native interface that meets engineering teams where they are, while exposing a Simulink-compatible data model for organisations that use model-based design workflows.

## 1.3  Scope

HRTF covers the following in scope:

- Scenario-based test authoring for bipedal humanoid robots

- Simulation back-ends: Gazebo (Harmonic), MuJoCo, and Isaac Sim (via bridge)

- Signal-level assertions over continuous time windows

- Baseline capture, storage, and regression comparison for learned policies

- Structured test reports in JSON, HTML, and PDF

- Future: hardware-in-the-loop (HIL) execution and ISO 25785-1 compliance mapping

Out of scope for all phases:

- Autonomous vehicle testing (separate domain)

- Manipulation-only arms without a locomotion component

- Real-time control design or code generation (not a design tool — a test tool)

## 1.4  Definitions

| Term | Definition |
| --- | --- |
| SUT | System Under Test — the humanoid robot (simulated or physical) being tested |
| Scenario | A named, parameterised test case describing initial conditions, disturbance inputs, duration, and pass/fail assertions |
| Signal assertion | A time-domain predicate applied to a logged telemetry signal (e.g., always_above, never_exceeds, reaches_within) |
| Baseline | A stored reference test result used for regression comparison |
| WBC | Whole-Body Controller — the Layer 2 control component that converts task-space targets to joint torques |
| HIL | Hardware-in-the-Loop — test execution mode where real actuator hardware replaces the simulated plant |
| SIL | Software-in-the-Loop — test execution using only software simulation |
| URDF | Unified Robot Description Format — XML format describing robot kinematics and dynamics |
| CoM | Centre of Mass |
| ZMP | Zero-Moment Point — a stability metric for bipedal locomotion |

2. Stakeholders and user roles

## 2.1  Primary stakeholders

| Stakeholder | Goal | Success measure |
| --- | --- | --- |
| Test / systems engineer | Author and run test suites without building simulation scaffolding from scratch | First test running in < 30 min from install |
| ML / controls engineer | Detect policy regressions before hardware deployment | Automated baseline diff on every model commit |
| Engineering manager | Evidence that the robot meets safety specs for customer / regulator | One-click compliance-style report |
| MathWorks product team | A validated beachhead in the humanoid robotics market | 3+ humanoid companies using HRTF within 12 months of release |
| Enterprise deployer (BMW, Amazon) | Assurance that vendor robots have been systematically validated | Structured test evidence attached to robot delivery |

3. System overview and architecture

## 3.1  High-level architecture

HRTF is structured as four layers:

- CLI / API layer — hrtf run, hrtf baseline, hrtf report commands; Python API for programmatic use

- Scenario layer — YAML-defined test scenarios referencing robot URDF, disturbance profile, and assertions

- Simulation adapter layer — pluggable adapters for Gazebo, MuJoCo, and Isaac Sim; uniform signal output format

- Reporting layer — JSON result files, HTML dashboards, and PDF test reports

## 3.2  Integration points

- ROS 2 (Humble / Jazzy): topic subscription for telemetry, service calls for scenario control

- GitHub Actions / GitLab CI: hrtf exits non-zero on test failure; JSON results parseable by any CI system

- Simulink (optional): MATLAB data import of .mat result files for further analysis in Simulink Test

- URDF / SDF: any robot describable in URDF is loadable without code changes

4. Requirements — priority legend

| Code | Label | Meaning |
| --- | --- | --- |
| P0 | MVP — must have | Required for the demo to be credible. Build first. |
| P1 | MVP — strong | Significantly improves demo quality. Build if time allows in MVP phase. |
| P2 | v1.0 — product | Required for a usable v1.0 product release. |
| P3 | v2.0 — vision | Future capability. Document now, build later. |

5. Functional requirements

## 5.1  Robot model loading and configuration

HRTF must support loading any humanoid robot model described in URDF or SDF without requiring code changes. The platform must validate the model on load and surface actionable errors.

| ID | Requirement | Priority | Phase |
| --- | --- | --- | --- |
| RL-01 | Load a robot from a URDF file path or ROS 2 robot_description parameter without writing Python code | P0 | MVP |
| RL-02 | Support Unitree G1, H1, and iCub URDFs as reference models included with the framework | P0 | MVP |
| RL-03 | Validate URDF on load and emit structured errors identifying the malformed element, line number, and suggested fix | P0 | MVP |
| RL-04 | Support SDF model format (Gazebo native) in addition to URDF | P1 | MVP |
| RL-05 | Allow joint limit overrides in scenario YAML without modifying the source URDF | P2 | v1.0 |
| RL-06 | Support robot model versioning — store model hash alongside test results so results are traceable to a specific URDF revision | P2 | v1.0 |
| RL-07 | Support parametric robot families where hardware variants share a base URDF with configuration overlays | P3 | v2.0 |

## 5.2  Scenario authoring

Test scenarios must be expressible as YAML files without requiring Python code for common cases. The YAML schema must be versioned and validated. All parameters must be overridable from the CLI for CI/CD use.

| ID | Requirement | Priority | Phase |
| --- | --- | --- | --- |
| SA-01 | Define a scenario as a YAML file specifying: robot model, simulator, initial pose, disturbance profile, duration, and assertions | P0 | MVP |
| SA-02 | Provide a hrtf init command that generates a valid scenario template for a given robot URDF | P0 | MVP |
| SA-03 | Validate scenario YAML against a published JSON Schema on load and emit line-level errors | P0 | MVP |
| SA-04 | Support parametric scenarios where numeric values (force magnitude, step height, duration) are overridable via CLI flags | P0 | MVP |
| SA-05 | Include a built-in library of 10 canonical humanoid scenarios (see Section 6) requiring no authoring to run | P0 | MVP |
| SA-06 | Support scenario inheritance: a scenario YAML may extend another and override specific fields | P1 | MVP |
| SA-07 | Allow scenario grouping into named test suites with shared setup and teardown | P2 | v1.0 |
| SA-08 | Provide a visual scenario editor (web UI) for authoring disturbance profiles without editing YAML directly | P3 | v2.0 |

## 5.3  Simulation execution

HRTF must support at least two simulation back-ends at MVP and provide a documented adapter interface for adding additional back-ends. Simulation must be deterministic given the same seed and scenario parameters.

| ID | Requirement | Priority | Phase |
| --- | --- | --- | --- |
| SE-01 | Execute scenarios in Gazebo Harmonic via the standard ROS 2 bridge without additional configuration | P0 | MVP |
| SE-02 | Execute scenarios in MuJoCo via the mujoco Python bindings | P0 | MVP |
| SE-03 | Guarantee deterministic simulation output for identical scenario YAML and random seed | P0 | MVP |
| SE-04 | Support parallel scenario execution across CPU cores with configurable worker count | P1 | MVP |
| SE-05 | Provide a documented SimulatorAdapter abstract base class enabling third-party back-end integration in < 200 lines of Python | P1 | MVP |
| SE-06 | Support Isaac Sim as an optional back-end via the Isaac Sim Python API | P2 | v1.0 |
| SE-07 | Support hardware-in-the-loop (HIL) execution mode where the simulated plant is replaced by real hardware over EtherCAT or CAN | P3 | v2.0 |
| SE-08 | Support cloud-parallel execution across multiple machines via a configurable executor back-end | P3 | v2.0 |

## 5.4  Signal logging and assertions

This is the core technical differentiator over pytest. HRTF must capture full telemetry at simulation frequency and provide a rich library of time-domain predicates for pass/fail evaluation.

| ID | Requirement | Priority | Phase |
| --- | --- | --- | --- |
| AS-01 | Automatically log all ROS 2 topics published by the SUT at native frequency for the scenario duration | P0 | MVP |
| AS-02 | Provide always_above(signal, value, window) assertion: signal must remain above value for the entire window | P0 | MVP |
| AS-03 | Provide never_exceeds(signal, value, window) assertion: signal must not exceed value at any sample in the window | P0 | MVP |
| AS-04 | Provide reaches_within(signal, value, tolerance, deadline) assertion: signal must reach value within tolerance before deadline | P0 | MVP |
| AS-05 | Provide stabilises_within(signal, tolerance, window) assertion: signal variance must fall below tolerance within window | P0 | MVP |
| AS-06 | On assertion failure, report: timestamp of first violation, signal value at violation, expected bound, and duration of violation | P0 | MVP |
| AS-07 | Support compound assertions combining multiple predicates with AND / OR logic | P1 | MVP |
| AS-08 | Provide event_occurs(event_name, deadline) assertion for discrete events (fall detected, task complete) | P1 | MVP |
| AS-09 | Support custom assertion functions defined in Python and referenced from scenario YAML | P2 | v1.0 |
| AS-10 | Provide a temporal logic assertion layer (LTL / MTL syntax) for advanced test authors | P3 | v2.0 |

## 5.5  Baseline management and regression testing

Regression testing after AI policy updates is the highest-value use case for ML engineers. HRTF must make baseline capture, storage, and comparison a first-class workflow requiring minimal manual steps.

| ID | Requirement | Priority | Phase |
| --- | --- | --- | --- |
| BL-01 | Capture a named baseline from a passing test run with hrtf baseline capture <run_id> <name> | P0 | MVP |
| BL-02 | Compare a new test run against a named baseline with hrtf baseline compare <run_id> <baseline_name> | P0 | MVP |
| BL-03 | Report per-metric regression delta (absolute and percentage) for all logged signals | P0 | MVP |
| BL-04 | Mark a comparison as REGRESSED if any metric degrades beyond a configurable tolerance (default 5%) | P0 | MVP |
| BL-05 | Store baselines as versioned JSON files compatible with git version control | P1 | MVP |
| BL-06 | Support baseline promotion workflow: automatically promote a new baseline when a run passes all assertions and regression checks | P2 | v1.0 |
| BL-07 | Display signal overlay plots comparing current run vs baseline for each assertion signal | P2 | v1.0 |
| BL-08 | Support fleet-level baseline aggregation across multiple physical robots to detect hardware-specific regressions | P3 | v2.0 |

## 5.6  Reporting

Reports must serve two audiences simultaneously: engineers who need signal-level detail and managers / customers who need a single pass/fail verdict with supporting evidence.

| ID | Requirement | Priority | Phase |
| --- | --- | --- | --- |
| RP-01 | Produce a machine-readable JSON result file for every test run containing: scenario metadata, per-assertion verdict, signal summary statistics, and run duration | P0 | MVP |
| RP-02 | Produce an HTML report with signal plots for each assertion, pass/fail badge, and scenario configuration | P0 | MVP |
| RP-03 | Exit with code 0 on all-pass and non-zero on any failure for CI/CD integration | P0 | MVP |
| RP-04 | Produce a one-page PDF executive summary showing overall pass/fail, scenario list, and critical signal plots | P1 | MVP |
| RP-05 | Include a test evidence manifest (scenario YAML, robot URDF hash, simulator version, random seed) sufficient to reproduce any result | P1 | MVP |
| RP-06 | Produce a SARIF-format report for integration with GitHub Advanced Security code scanning UI | P2 | v1.0 |
| RP-07 | Generate a requirements traceability matrix mapping test assertions to structured requirements | P2 | v1.0 |
| RP-08 | Generate an ISO 25785-1 compliance evidence package mapping applicable test scenarios to standard clauses | P3 | v2.0 |

## 5.7  CI/CD integration

| ID | Requirement | Priority | Phase |
| --- | --- | --- | --- |
| CI-01 | Provide a hrtf run CLI command executable in a headless Linux environment with no display server | P0 | MVP |
| CI-02 | Publish an official Docker image containing HRTF, ROS 2 Humble, and Gazebo Harmonic with a one-line run command | P0 | MVP |
| CI-03 | Provide a GitHub Actions workflow template (hrtf.yml) users can copy into their repo in under 5 minutes | P1 | MVP |
| CI-04 | Support GitLab CI via the same Docker image with a documented .gitlab-ci.yml snippet | P2 | v1.0 |
| CI-05 | Provide a webhook notification system posting test results to Slack or Teams channels on completion | P2 | v1.0 |
| CI-06 | Support test result caching: skip re-running a scenario if the URDF hash, scenario YAML hash, and policy hash are unchanged since last run | P3 | v2.0 |

6. Built-in scenario library

The following ten canonical scenarios must ship with HRTF at MVP. They are the primary evidence that the platform addresses real humanoid testing pain. Each scenario runs out-of-the-box on any URDF-described bipedal robot.

| ID | Scenario | Pass criterion | Primary signal | Priority |
| --- | --- | --- | --- | --- |
| SC-01 | Static balance | CoM height > 0.75 m, ZMP within support polygon for 30 s with no disturbance | CoM height, ZMP x/y | P0 |
| SC-02 | Lateral push recovery | CoM height > 0.75 m for 10 s after 50 N lateral impulse (200 ms) | CoM height, ankle torque | P0 |
| SC-03 | Flat-ground gait — 5 m walk | Reaches 5 m within 12 s; no fall; all joint torques within rated limits | Base velocity, CoM height | P0 |
| SC-04 | Joint torque compliance | No joint exceeds 95% of rated torque at any point during scenario SC-03 | Per-joint torque | P0 |
| SC-05 | Stair ascent — 3 steps | Robot CoM height increases monotonically; task completes within 20 s; no fall | CoM z, step contact events | P0 |
| SC-06 | Fall detection and e-stop | E-stop signal fires within 100 ms of CoM height < 0.3 m | E-stop latency, CoM height | P1 |
| SC-07 | Payload walking — 5 kg | Same as SC-03 with 5 kg payload attached; torque headroom > 10% | CoM height, joint torques | P1 |
| SC-08 | Slope ascent — 10 degrees | Robot walks 5 m up 10° incline without fall; completes within 15 s | CoM height, foot normal forces | P1 |
| SC-09 | Object grasp — cylindrical, 1 kg | Grasp success within 5 s; object not dropped for 10 s; wrist force within limits | Wrist force/torque, grasp event | P2 |
| SC-10 | Push recovery during manipulation | Robot maintains grasp during 30 N lateral push; no fall; recovers stability within 5 s | CoM height, wrist force, grasp event | P2 |

7. Non-functional requirements

## 7.1  Performance

| ID | Requirement | Priority | Phase |
| --- | --- | --- | --- |
| PF-01 | Complete scenario SC-01 (static balance, 30 s sim) in under 60 s wall-clock time on a standard laptop (8-core CPU, 16 GB RAM) using Gazebo Harmonic | P0 | MVP |
| PF-02 | Scale to 10 scenarios in parallel on a 16-core machine without inter-scenario interference | P1 | MVP |
| PF-03 | Log telemetry with zero sample loss at simulator frequencies up to 1 kHz | P1 | MVP |
| PF-04 | Generate an HTML report for a 10-scenario run in under 10 s | P2 | v1.0 |
| PF-05 | Support distributed execution across a 10-node cluster completing 100 scenarios in under 5 minutes | P3 | v2.0 |

## 7.2  Reliability and reproducibility

| ID | Requirement | Priority | Phase |
| --- | --- | --- | --- |
| RB-01 | Identical scenario YAML and random seed must produce bit-identical results across runs on the same machine and OS | P0 | MVP |
| RB-02 | A failed scenario must never corrupt the result store or prevent subsequent scenarios from running | P0 | MVP |
| RB-03 | Simulator crashes must be caught, the scenario marked ERROR (not PASS or FAIL), and a full stack trace included in the result JSON | P0 | MVP |
| RB-04 | The framework must recover automatically from a single simulator restart without human intervention | P2 | v1.0 |

## 7.3  Usability

| ID | Requirement | Priority | Phase |
| --- | --- | --- | --- |
| UX-01 | A robotics engineer with ROS 2 experience but no prior HRTF experience must be able to run their first scenario in under 30 minutes from a fresh install | P0 | MVP |
| UX-02 | All CLI commands must include --help output with concrete examples | P0 | MVP |
| UX-03 | Error messages must state what went wrong, which file and line caused it, and what the user should do to fix it | P0 | MVP |
| UX-04 | Assertion failures must display the failing signal value, the expected bound, and the timestamp of first violation — not just FAIL | P0 | MVP |
| UX-05 | The project README must include a complete quickstart running SC-01 on the Unitree G1 URDF in a Docker container | P0 | MVP |
| UX-06 | Provide a browser-based test result explorer for navigating multi-scenario runs without opening individual HTML files | P3 | v2.0 |

## 7.4  Extensibility

| ID | Requirement | Priority | Phase |
| --- | --- | --- | --- |
| EX-01 | The SimulatorAdapter interface must be documented and stable across minor versions (semantic versioning enforced) | P1 | MVP |
| EX-02 | Custom assertion functions must be registerable via a Python entry-point without modifying framework source | P2 | v1.0 |
| EX-03 | Custom scenario scenario types must be registerable via a Python entry-point | P2 | v1.0 |
| EX-04 | The result JSON schema must be versioned and backward-compatible within a major version | P2 | v1.0 |
| EX-05 | Provide a Simulink data import utility converting HRTF result JSON to .mat format for analysis in MATLAB / Simulink Test | P2 | v1.0 |

## 7.5  Security and data handling

| ID | Requirement | Priority | Phase |
| --- | --- | --- | --- |
| SC-SEC-01 | Test result files must include a SHA-256 hash of the scenario YAML and robot URDF to detect tampering | P1 | MVP |
| SC-SEC-02 | The Docker image must be published to a public registry with signed images and a published SBOM | P2 | v1.0 |
| SC-SEC-03 | Sensitive robot model data (proprietary URDFs) must never be transmitted outside the user's machine | P0 | MVP |

8. Constraints and assumptions

## 8.1  Technical constraints

- Target OS: Ubuntu 22.04 LTS and Ubuntu 24.04 LTS. macOS and Windows are explicitly out of scope for MVP.

- Python version: 3.10 or later

- ROS 2 distribution: Humble Hawksbill (LTS) with optional support for Jazzy Jalisco

- All dependencies must be installable via pip and apt with no proprietary licenses for core functionality

- Simulink integration is optional and must degrade gracefully when MATLAB is not installed

## 8.2  Assumptions

- Users have a working ROS 2 installation and basic familiarity with ROS 2 concepts (topics, nodes, URDF)

- The robot's control stack publishes standard ROS 2 messages (sensor_msgs/JointState, geometry_msgs/Twist, nav_msgs/Odometry)

- Physics simulation is the primary test environment at MVP; hardware-in-the-loop is a future phase

- ISO 25785-1 is not yet published; compliance mapping will be added when the standard is finalised (expected 2026–2027)

9. Delivery phases

| Phase | Timeline | Deliverables | Success gate |
| --- | --- | --- | --- |
| MVP demo | 0–3 months | 5 canonical scenarios (SC-01 to SC-05), Gazebo + MuJoCo adapters, CLI runner, JSON + HTML reports, Docker image, GitHub Actions template, Unitree G1 quickstart | SC-01 to SC-05 all pass on G1 URDF in Docker |
| MVP complete | 3–5 months | All 10 canonical scenarios, baseline management, PDF report, MuJoCo determinism verified, public GitHub repo with CI passing, README quickstart < 30 min | Two external engineers successfully run quickstart independently |
| v1.0 product | 6–12 months | Isaac Sim adapter, signal overlay plots, requirements traceability matrix, SARIF output, custom assertion API, Simulink .mat export, semantic-versioned API | At least one humanoid company uses HRTF in their CI pipeline |
| v2.0 vision | 12–24 months | HIL execution mode, ISO 25785-1 compliance package, fleet-level baseline aggregation, distributed cloud executor, web-based result explorer | MathWorks product team adopts HRTF as a supported toolbox |

10. Open questions and decisions required

OQ-01 — Naming The working name HRTF is a placeholder. Consider a name that communicates test-and-validation without acronym collision (HRTF is also an audio term). Decide before public GitHub launch.

OQ-02 — License MIT license maximises adoption by commercial robotics companies. Apache 2.0 provides explicit patent protection. GPL would prevent commercial adoption. Recommend MIT or Apache 2.0. Decide before first public commit.

OQ-03 — Scenario pass/fail thresholds Current thresholds (e.g., CoM > 0.75 m, 50 N push) are reasonable defaults but not grounded in a published standard. Validate with at least two humanoid engineers before locking the built-in scenario library.

OQ-04 — MathWorks relationship If MathWorks adopts HRTF, the Simulink .mat export (EX-05) becomes a strategic integration point. Clarify whether this should be a bidirectional bridge (Simulink can also trigger HRTF scenarios) before committing to the API design.

OQ-05 — ROS 2 message assumptions The built-in scenarios assume robots publish sensor_msgs/JointState and geometry_msgs/Pose for CoM. Some companies use custom message types. Decide whether to require a thin adaptation layer or ship adapters for known robots (Unitree, Agility Digit).

Appendix A — Requirement index

All requirement IDs, phases, and priorities at a glance.

| ID | Requirement | Section | Phase | Priority |
| --- | --- | --- | --- | --- |
| RL-01 | Load robot from URDF / robot_description | 5.1 | MVP | P0 |
| RL-02 | Unitree G1, H1, iCub reference URDFs | 5.1 | MVP | P0 |
| RL-03 | URDF validation with structured errors | 5.1 | MVP | P0 |
| RL-04 | SDF format support | 5.1 | MVP | P1 |
| RL-05 | Joint limit overrides in scenario YAML | 5.1 | v1.0 | P2 |
| RL-06 | URDF hash traceability | 5.1 | v1.0 | P2 |
| RL-07 | Parametric robot family support | 5.1 | v2.0 | P3 |
| SA-01 | YAML scenario definition | 5.2 | MVP | P0 |
| SA-02 | hrtf init template generation | 5.2 | MVP | P0 |
| SA-03 | YAML JSON Schema validation | 5.2 | MVP | P0 |
| SA-04 | Parametric CLI overrides | 5.2 | MVP | P0 |
| SA-05 | 10 built-in canonical scenarios | 5.2 | MVP | P0 |
| SA-06 | Scenario inheritance | 5.2 | MVP | P1 |
| SA-07 | Named test suites with setup/teardown | 5.2 | v1.0 | P2 |
| SA-08 | Visual scenario editor | 5.2 | v2.0 | P3 |
| SE-01 | Gazebo Harmonic adapter | 5.3 | MVP | P0 |
| SE-02 | MuJoCo adapter | 5.3 | MVP | P0 |
| SE-03 | Deterministic execution with seed | 5.3 | MVP | P0 |
| SE-04 | Parallel scenario execution | 5.3 | MVP | P1 |
| SE-05 | SimulatorAdapter ABC | 5.3 | MVP | P1 |
| SE-06 | Isaac Sim adapter | 5.3 | v1.0 | P2 |
| SE-07 | HIL execution mode | 5.3 | v2.0 | P3 |
| SE-08 | Cloud-parallel executor | 5.3 | v2.0 | P3 |
| AS-01 | Full ROS 2 topic logging | 5.4 | MVP | P0 |
| AS-02 | always_above assertion | 5.4 | MVP | P0 |
| AS-03 | never_exceeds assertion | 5.4 | MVP | P0 |
| AS-04 | reaches_within assertion | 5.4 | MVP | P0 |
| AS-05 | stabilises_within assertion | 5.4 | MVP | P0 |
| AS-06 | Assertion failure detail reporting | 5.4 | MVP | P0 |
| AS-07 | Compound AND/OR assertions | 5.4 | MVP | P1 |
| AS-08 | event_occurs assertion | 5.4 | MVP | P1 |
| AS-09 | Custom Python assertion functions | 5.4 | v1.0 | P2 |
| AS-10 | Temporal logic (LTL/MTL) assertions | 5.4 | v2.0 | P3 |
| BL-01 | Baseline capture command | 5.5 | MVP | P0 |
| BL-02 | Baseline compare command | 5.5 | MVP | P0 |
| BL-03 | Per-metric regression delta report | 5.5 | MVP | P0 |
| BL-04 | REGRESSED verdict on tolerance breach | 5.5 | MVP | P0 |
| BL-05 | Git-compatible JSON baseline storage | 5.5 | MVP | P1 |
| BL-06 | Automatic baseline promotion | 5.5 | v1.0 | P2 |
| BL-07 | Signal overlay plots vs baseline | 5.5 | v1.0 | P2 |
| BL-08 | Fleet-level baseline aggregation | 5.5 | v2.0 | P3 |
| RP-01 | JSON result file | 5.6 | MVP | P0 |
| RP-02 | HTML report with signal plots | 5.6 | MVP | P0 |
| RP-03 | CI exit codes | 5.6 | MVP | P0 |
| RP-04 | PDF executive summary | 5.6 | MVP | P1 |
| RP-05 | Test evidence manifest | 5.6 | MVP | P1 |
| RP-06 | SARIF format output | 5.6 | v1.0 | P2 |
| RP-07 | Requirements traceability matrix | 5.6 | v1.0 | P2 |
| RP-08 | ISO 25785-1 compliance package | 5.6 | v2.0 | P3 |
| CI-01 | Headless CLI runner | 5.7 | MVP | P0 |
| CI-02 | Official Docker image | 5.7 | MVP | P0 |
| CI-03 | GitHub Actions template | 5.7 | MVP | P1 |
| CI-04 | GitLab CI support | 5.7 | v1.0 | P2 |
| CI-05 | Slack/Teams webhook notifications | 5.7 | v1.0 | P2 |
| CI-06 | Result caching by hash | 5.7 | v2.0 | P3 |

End of document HRTF — Humanoid Robot Test Framework ¦ Requirements Specification v1.0 ¦ April 2026